Using PLR to create confidence intervals for an error matrix exercise

In this exercise we will use PLR to generate probabilistic confidence intervals for individual classes and overall map accuracy. To do this we will be using the results from exercise 3. If you have not finished exercise 3 please do so before starting this exercise (link to exercise 3).

To begin the exercise, please open the “AccuacyAssessment.mxd” map document (link to map document). If the “Training” and “Validation” point feature classes are not already loaded into the map please add them at this time. “Training” and “Validation” feature classes can be found in the “AccuracyAssessment.gdb” file geodatabase and are named “Training” and “Validation”, respectively. Next, let’s open each of the SAS outputs files (prc.lst) generated in exercise 3. The accuracy assessment output associated with the training dataset should be located within the folder SASOUTPUT\ACCURACYASSESSMENT while the results from the “Validation” dataset should be located within the folder SASOUTPUT\ValidationAA. After opening each respective result, scroll down to “The Logistic Procedure” section of each output (Validation and Training datasets displayed left to right below).

The first section of the logistic procedure output describes the data used for the analysis and how the logits are organized (note the total weight column). After reviewing the initial model setup, scroll down to the section on model fit statistics.

Both the validation and Training datasets models are statistically significant (LL p-value < 0.0030, blue box) and have similar Max-rescaled R-Square values, suggesting that the original classification is not bias.

Using the Maximum Likelihood parameter estimates and their associated estimates of error we can calculate the probability (user probability) of each category occurring given one of our mapped classes. Using those probabilities we can create an error matrix of user and producer probabilities with confidence intervals for each class. To illustrate let’s look closer at the results from our validation dataset. Scroll down to the “Analysis of Maximum Likelihood Estimates” section of the output. Look at the label headings Parameter (blue outline), Category (red outline), Estimate (green outline), and Standard Error (brown outline). The Parameter column identifies either the intercept of each logit or the corresponding mapped class beta. The Category column defines which beta estimate applies to a given logit equation. The Estimate column depicts the maximum likelihood estimate of the beta. The Standard Error column provides an estimate of error related to the beta.

To estimate the probability of each category given a mapped class lets rearrange these results into a series of linear equations that estimate the natural log odds of a given Category.

- LNπ1π14=1.9098+0.48811-1.50435-1.68666-1.909810+13.086011-3.519212

- LNπ5π14=1.8133-2.50651+0.53815-3.19966-1.407810+13.470111-2.729612

- LNπ6π14=1.7913-1.38581-2.48445-0.40506-1.385810+12.799011-2.302112

- LNπ10π14=-0.8182+0.12501+0.12505-0.56816+2.897610+14.715311-15.761512

- LNπ11π14=1.7334-2.42651-1.73345-2.02106-1.733410+14.936411-2.244212

- LNπ12π14=-4.2675-9.76941-9.60815-11.78776+4.267510+19.550911+5.430712

Subscripts after each beta estimate denote the slope for each given mapped class. For a given mapped pixel only 1 beta estimate is applicable. This beta estimate is multiplied by 1 while the rest of the estimates are multiplied by 0 negating their effect on the model (i.e. dummy variables). For example, let’s assume we have a pixel with a map class of 5. The logits simplify down to the following:

- LNπ1π14=0.4055

- LNπ5π14=2.3514

- LNπ6π14=-0.6931

- LNπ10π14=-0.6932

- LNπ11π14=0

- LNπ12π14=-13.8756

To calculate probability we manipulate the logits as follows:

- π1=e0.40551+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.100

- π5=e2.35141+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.700

- π6=e-0.69311+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.033

- π10=e-0.69321+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.033

- π11=e01+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.067

- π12=e-13.87561+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.000

- π14=11+e0.4055+e2.3514+e-0.6931+e-0.6932+e0+e-13.8756=0.067

To generate a confidence intervals around each class’ predicted probabilities we us the delta method to estimate  σ2(πi) and solve as follows πi±zα/2σπi. Fortunately, user predicted, lower and upper confidence limits probabilities are calculated for you using the “Accuracy Assessment” tool. To view these estimates open the “CCCD” attribute table by right clicking on the raster layer (note, estimates are using the weighted training data not the weighted validation data and may vary from the results explained above).

After opening the CCCD attribute table you should notice a number of new fields. Fields with the prefix “P” denote the predicted probabilities (user probabilities) of each field verified category. Categories can be identified following the underscore of each field name. A prefix of “LCL” denotes the lower confidence limit while a prefix of “UCL” denotes the upper confidence limit of user probabilities. The final step to creating an error matrix with confidence intervals is to multiply each user probability by its corresponding abundance (the Count Field), thereby estimating each cell’s predicted lower and upper frequency. This can be done in a number of different software packages including ESRI, but is probably easier to manipulate in a spreadsheet program such as Excel. To export the “CCCD” table to Excel left click the table properties dropdown and click on Export.

Next, click on the open output button and navigate to the “AccuracyAssessment” directory. Type in “AccuracyAssessmentResults.csv” and click the Save button. Finally, click on the OK button of the “Export Data” form. When prompted don’t open the table in the current map. Instead open this file with Microsoft Excel (remember to select .csv as the extension) and multiply each probability by its corresponding Count to produce the following frequency table.

| Upper |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Predict | 1 | 5 | 6 | 10 | 11 | 12 | 14 |  |
| Lower |  |  |  |  |  |  |  |  |
|  | 46,689 | 5,314 | 5,314 | 15,887 | 6 | 20,264 | 8,390 | 101,864 |
| 1 | 35,298 | 1,616 | 1,616 | 8,079 | 0 | 11,311 | 3,232 | 61,152 |
|  | 23,908 | 0 | 0 | 272 | 0 | 2,358 | 0 | 26,538 |
|  | 8,587 | 36,455 | 9,521 | 20 | 12,255 | 6,651 | 5,090 | 78,579 |
| 5 | 2,957 | 26,016 | 3,465 | 0 | 5,082 | 1,982 | 1,290 | 40,793 |
|  | 0 | 15,578 | 0 | 0 | 0 | 0 | 0 | 15,578 |
|  | 48,042 | 48,957 | 330,907 | 48,042 | 43,625 | 62,609 | 37,866 | 620,049 |
| 6 | 30,914 | 31,643 | 296,824 | 30,914 | 27,429 | 42,735 | 22,972 | 483,431 |
|  | 13,786 | 14,328 | 262,742 | 13,786 | 11,233 | 22,862 | 8,078 | 346,812 |
|  | 232,471 | 310,136 | 274,907 | 1,897,000 | 214,426 | 235,340 | 297,910 | 3,462,189 |
| 10 | 189,997 | 260,967 | 228,626 | 1,809,618 | 173,701 | 192,596 | 249,720 | 3,105,227 |
|  | 147,527 | 211,802 | 182,346 | 1,722,237 | 132,979 | 149,856 | 201,530 | 2,748,276 |
|  | 97,102 | 88,767 | 76,962 | 69,513 | 598,238 | 61,302 | 120,849 | 1,112,733 |
| 11 | 71,762 | 64,620 | 54,632 | 48,417 | 551,755 | 41,657 | 92,440 | 925,283 |
|  | 46,423 | 40,474 | 32,301 | 27,320 | 505,273 | 22,014 | 64,031 | 737,835 |
|  | 354,785 | 437,987 | 415,471 | 401,639 | 400,659 | 2,977,659 | 483,157 | 5,471,358 |
| 12 | 302,640 | 380,033 | 359,015 | 346,127 | 345,218 | 2,869,184 | 422,340 | 5,024,558 |
|  | 250,495 | 322,075 | 302,559 | 290,616 | 289,777 | 2,760,704 | 361,523 | 4,577,748 |
|  | 8,493 | 5,506 | 5,506 | 22 | 13,319 | 10,171 | 33,327 | 76,344 |
| 14 | 2,797 | 1,398 | 1,398 | 0 | 5,594 | 3,703 | 22,710 | 37,600 |
|  | 0 | 0 | 0 | 0 | 0 | 0 | 12,093 | 12,093 |
|  | 796,169 | 933,122 | 1,118,588 | 2,432,123 | 1,282,526 | 3,373,997 | 986,590 | 10,923,115 |
|  | 636,365 | 766,294 | 945,578 | 2,243,156 | 1,108,778 | 3,163,169 | 814,703 | 9,678,043 |
|  | 482,137 | 604,257 | 779,948 | 2,054,230 | 939,262 | 2,957,793 | 647,254 | 8,464,881 |

Next, we convert these counts along the diagonal into proportions by summing Upper, Predict, and Lower counts and dividing them by their row or column totals, producing confidence limits on map accuracy (54.2% to 62.64%; α = 0.05). Moreover we can apply this same strategy for individual cells within our error matrix to estimate misclassification error among classes with a predefined level of confidence. Note that user accuracy with confidence limits are already calculated within the attribute table of the CCCD raster surface. Produce error can be calculated with an upper and lower confidence limit by dividing cell counts by their column totals. For example, if we wanted to know producer accuracy for mapped class 11 and category 6, meaning when a given pixel is class 6 on the ground what is the probability that we mapped it as 11, we would divide 54,632 by 945,578, giving us a producer error of 5.78% (4.14% to 6.88%; 95% CI).

Exercise Questions [Answers]

- Why is it important to produce confidence intervals for map accuracy?

- How would you interpret the user error between mapped class 12 and on the ground categories 5 and 14 (i.e., is there a difference between misclassification rates)?

- How could confidence intervals be used to determine if mapped classes are different?

You may also want to look at the original CLUST raster surface to see the distribution of each cluster. It is also located in the AccuracyAssessment field geodatabase.

If you ran additional Accuracy Assessments and renamed the output files then please navigate to that location instead of the default location.

Max rescaled r-square is interpreted as the amount of information gained from intercept alone model. Values range from 0-1. For a more thorough description please refer to SAS help.

Odds are interpreted as the probability of a given class divided by the probability of a baseline category (in this example  the baseline category is 14).