Accuracy assessment for a subset of a map exercise

In this exercise we will extract a portion of the map, allocate new plots for that map, perform a new accuracy assessment, and then compare and contrast these results to our validation results in exercise 4. If you have not finished exercise 4 please do so before starting this exercise (link to exercise 4).

To begin the exercise, please open the “AccuacyAssessment.mxd” map document (link to map document). If the “CCCD” raster surface is not loaded into the map please add it at this time. Next, let’s open the “Validation” SAS outputs files (prc.lst) generated in exercise 4. The accuracy assessment output associated with the “Validation” dataset should be located within the folder SASOUTPUT\ValidationAA. Next, please load the polygon feature class called study area extent into the map. “StudyAreaExtent” defines the boundary of the area that we want to reevaluate the accuracy of the map. To subset the full map open the “Clip Raster” form by click on the “Clip Analysis” button located within the “Raster Analysis” menu of the “RMRS Raster” Utility toolbar.

Select the “StudyAreaExtent” polygon for the “Clipping Polygon” dropdown. Select the “CCCD” for the “Raster to Clip” dropdown. In the “Out Raster Name" textbox type “CCCD_subset”. Leave “Erase” unchecked. Click on the “Clip” button. This will produce a function dataset called CCCD_subset. To make a permanent raster dataset open the “Save Raster” form by click on the save raster button  of the “RMRS Raster Utility” toolbar.

In the “Raster to Save” dropdown select “CCCD_subset”. Select the “AccuracyAssessment.gdb” file geodabase using the open button  and navigating to the “AccuracyAssessment.gdb” database located within this exercise. Type in “subCCCD” in the “Out Name” textbox and click “Save”. When prompted “Do you want to add raster to current map, click on the “Yes” button. Next close the running process dialog and make your “subCCCD” layer visible. Open the raster attribute table by right clicking on the newly created layer and clicking on open attribute table. Notice that there are only 3 fields; “OBJECTID”, “Value”, and “Count”. The “Value” field has the values associated with map classes and the “Count” field has the updated cell counts associated with each classes.

Let’s added descriptive names to this attribute table by adding a field named “Description”, performing a join with the attribute table of “CCCD”, and populating the “Description” field with the values in the 
“Description” field of the “CCCD” attribute table. Left click on the “subCCCD” raster attribute table properties dropdown (circled in red above) and left click add field. Name the field “Description” and specify the type as text. Set the length to 25 and click the ok button. Next, right click the raster surface layer “subCCCD” (the top “CCCD_subset”) left click the “Joins and relate” button, and left click the “Join” button.

Select the “Value” field from the “Choose the field I this layer that the join will be based on:” dropdown. Select the “CCCD” raster layer from the “Choose the table to join to this layer, or load the table from disk:” dropdown. Select the “Value” field from the “Choose the field in the table to base the join on:” dropdown. Make sure the Keep all records toggle is selected and click the ok button. If prompted to create an index select yes. This will add all the fields and values that match between the two tables based on the value field into the “subCCCD” raster attribute table.

Next, right click on the field   “Vat_subCCCD.description” and click on the “Field Calculator” button to open the “Field Calculator” form.

In the Fields listbox left double click the VAT_CCCD.Description field and then click ok. Next remove the join and you should now have a descriptive field in your attribute table.

Next, add the “subValidation” dataset to your map and perform an accuracy assessment as before using the “Value” field, the “Category” field, and the “Weight” field. Finally, create the output error matrix as described in exercise 4.

Exercise Questions [Answers]

- What is map accuracy (90% CI) of the subCCCD raster surface?

- What is the map accuracy (95% CI) of the subCCCD raster surface?

- Is there a statistical difference in map accuracy between the full map and the subset?

If you ran additional Accuracy Assessments and renamed the output files then please navigate to that location instead of the default location.